Purpose

This report serves as an accounting of all actions that took place on that date to the right as recorded by your AFCO Customer Care Specialist.

| Visit Outline |  |
| --- | --- |
| Report Date : | 01/09/2025 |
| No Of Days : | 1 |
| AFCO CCS : | Kyle Miller |
| Client : | Kuna Foodservice - Dupo, IL |
| Client Contact : | Jim Patterson |

| Topic Name Equipment | Priority Normal Priority | Status Open |
| --- | --- | --- |
| Both dosatrons were observed The East Dosatron for the 386L was titrating very low Initially made adjustments to the unit, with no success Discovered the pickup line had air bubbles in it from the jug swap Purged out air bubbles and titration was in range The West Dosatron was working properly with no adjustments needed The Knight pump for Odorless LBA lost prime due to the bucket being empty Replaced bucket with a new full one Primed pump and chemical was flowing through the pump properly |  |  |
| Responsibility: AFCO,Plant |  |  |

| Topic Name General | Priority Normal Priority | Status Open |
| --- | --- | --- |
| Time at facility 1-9-25 0810 - 0930 Met with Plant Engineer/Fleet Manager and Safety Manager/SQF Practitioner to discuss: Inventory and ordering Order 4008976134 was placed in the system Chemical container disposal Transferred remaining chemicals from Pro Chlor, 386L, and Odorless LBA to new containers to free up floor space Informed the plant the containers are ready for specific disposal procedures Infobase login for customer Sending information to the plant as soon as I get it No further issues or concerns by the plant |  |  |
| Responsibility: AFCO,Plant |  |  |

| Topic Name Inventory | Priority Normal Priority | Status Open |
| --- | --- | --- |
| Inventory report and spend for 12-10-24 to 1-9-25 No major variances to report Order 4008976134 2 - Amine Z 2 - Odorless LBA 1 - Pro Chlor 1 - 386L Case 1 - Zep Klear |  |  |
| Responsibility: AFCO,Plant |  |  |

| Topic Name Safety | Priority Normal Priority | Status Open |
| --- | --- | --- |
| Safety scan is attached Chemical room was organized Labels were in place and in good condition |  |  |
| Responsibility: AFCO,Plant |  |  |

| Topic Name Titrations | Priority Normal Priority | Status Open |
| --- | --- | --- |
| Titrations: Target Ranges: Amine Z Product contact: 200 - 400 ppm Environmental: 400 - 800 ppm Formula 386L Product contact: 200 - 400 ppm Environmental: 400 - 800 ppm Pro - Chlor Manual / foaming: 1 - 6+ ounces per gallon of water *depending on the level of soil present West Dosatron Amine Z: 290 ppm East Dosatron 386L: 320 ppm Water temperature: 134F The Kuna titration log in the red folder was completed by the Zep account rep and left with the Production Manager |  |  |
| Responsibility: AFCO,Plant |  |  |

# Plant: Kuna Foodservice - Dupo, IL Date: 01/09/2025

Safety Scan	AFCO-Zep Corporate Office

5121 Coffey Ave

Chambersburg, PA 17201

Note: Imminent safety hazards should be corrected immediately!

| No. | Question | Yes | No | N/A | Repeat | Corrective Action |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Was PPE being used during sanitation process: |  |  | ✓ |  | Sanitation not in progress at the time of visit. |
|  | Face & eye protection? |  |  | ✓ |  |  |
|  | Boots? |  |  | ✓ |  |  |
|  | Wet suit or apron / sleeves? |  |  | ✓ |  |  |
|  | Gloves? |  |  | ✓ |  |  |
|  | Hearing protection? |  |  | ✓ |  |  |
| 2 | Are chemical use containers and secondary containers adequately labeled? | ✓ |  |  |  |  |
| 3 | Are SDS up to date (most recent)? (Vendor focus) | ✓ |  |  |  |  |
| 4 | Are eye wash stations & emergency showers easily accessible - not blocked? | ✓ |  |  |  |  |
| 5 | Are Chemical Safety Wall Charts posted in dispensing and storage locations? | ✓ |  |  |  |  |
| 6 | Are chemicals being used in a safe manner, guidelines being followed? | ✓ |  |  |  |  |
| 7 | Are sanitation tools in good repair; hoses, foamers, buckets, etc. | ✓ |  |  |  |  |
| 8 | Are incompatible chemicals adequately separated? (In use areas and storage) | ✓ |  |  |  |  |
| 9 | Is chemical storage area secured or set-up for security? | ✓ |  |  |  |  |
| 10 | Is spill containment in place or available? | ✓ |  |  |  |  |
| 11 | Are bulk chemical unloading ports labeled and locked? |  |  | ✓ |  | No bulk received at the plant. |
| 12 | Are slip hazards identified & kept under control during sanitation? |  |  | ✓ |  |  |
| 13 | Are wet floors marked slippery where necessary? | ✓ |  |  |  |  |
| 14 | Was LOTO policy being followed during Sanitation? |  |  | ✓ |  |  |

# COP / CIP / ACS

| 15 | Are cleaning pipe lines, chemical dosing pumps & lines adequately labeled? | ✓ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 16 | Are there any leaks; i.e.: steam valves, solution pipe lines, chemical dosing pumps or lines, etc. |  | ✓ |  |  |  |

Inventory Cost Sheet	AFCO-Zep Corporate Office

5121 Coffey Ave

Chambersburg, PA 17201

| Technical Representative: Kyle Miller | Date: 01/09/2025 | Plant: Kuna Foodservice - Dupo, IL |
| --- | --- | --- |
| Control Period Service Dates: 12/10/2024 - 01/09/2025 | Production Days: 30 | Location: |
| Country: |  |  |

| Product | Begin | Rcvd | End | Used | Avg Daily Use | Avg Daily Cost | Use/Day Last Pd | Days On Hand | Unit Cost | Tot Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 090085 Zep Alcohol Hand Sanitizer Spray DR55GAL | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | $0.00 | 0.00 | 0.00 | $19.00 | $0.00 |
| 126950 A-One DR20GAL | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | $0.00 | 0.00 | 0.00 | $16.50 | $0.00 |
| 166839 Zep Odorless LBA DR5GAL | 10.00 | 0.00 | 5.00 | 5.00 | 0.17 | $4.07 | 0.00 | 30.00 | $24.41 | $122.03 |
| 170039 FS Amine Z DR5GAL | 5.00 | 0.00 | 4.00 | 1.00 | 0.03 | $0.67 | 0.06 | 120.00 | $20.18 | $20.18 |
| 241285 FS Pro-Chlor DR55GAL | 45.00 | 0.00 | 40.00 | 5.00 | 0.17 | $3.07 | 0.47 | 240.00 | $18.39 | $91.96 |
| 338624 FS Formula 386L CS4X1GAL | 5.00 | 0.00 | 1.00 | 4.00 | 0.13 | $3.43 | 0.05 | 7.50 | $25.71 | $102.82 |
| 354424 ZEP FS Formula 4169 4x1gl | 6.00 | 0.00 | 6.00 | 0.00 | 0.00 | $0.00 | 0.06 | 0.00 | $0.00 | $0.00 |
| AF5249E Scrubbie DR5GAL | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | $0.00 | 0.00 | 0.00 | $21.71 | $0.00 |
| AF5266E Safe Clean DR5GAL | 1.00 | 0.00 | 0.00 | 1.00 | 0.03 | $0.42 | 0.03 | 0.00 | $12.71 | $12.71 |
| Zep Formula 4489 CS4X1GAL | 8.50 | 0.00 | 8.00 | 0.50 | 0.02 | $0.00 | 0.00 | 480.00 | $0.00 | $0.00 |
| Zep FS Formula 10184 4- 1GL | 11.00 | 0.00 | 10.00 | 1.00 | 0.03 | $0.00 | 0.06 | 300.00 | $0.00 | $0.00 |

| Product | Begin | Rcvd | End | Used | Avg Daily Use | Avg Daily Cost | Use/Day Last Pd | Days On Hand | Unit Cost | Tot Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Explanation of Variance from Projection: |  |  |  |  |  | Total Cost: $349.70 Daily Cost: $11.66 Daily Cost last period: $11.48 Difference Cost: $0.17 |  |  |  |  |