PRODUCT SPECIFICATIONS FOR

IDAHOAN® FLAKES

WIP #2100042

Issued: October 29, 2025

Supersedes: May 2, 2024

I. 	PRODUCT DESCRIPTION

Idahoan® Potato Flakes are processed from sound, well peeled and trimmed 100% Idaho® Potatoes. The product complies with the provisions of the Federal Food, Drug, and Cosmetic Act and Amendments.

II.   	INGREDIENTS

Idaho® potatoes with emulsifier (monoglycerides) and preservative (sodium acid pyrophosphate, sodium bisulfite, citric acid, mixed tocopherols).

III.  	COOKED PRODUCT REQUIREMENTS

Shall be precooked, cooled and cooked. Product must undergo cooling process to meet production specifications. Refer to R&D Processing Parameters for more information. Cooked product shall contain soft, tender lumps. Product is easily mashed, not sticky or pasty and no raw or undercooked slabs.

IV.  	PRODUCTION FEED RATES & TARGETS

| LEWISVILLE PLANT |  |  |
| --- | --- | --- |
| Additives | Item Number | Batch Weights |
| Tank #1 Emulsifier |  | 60 Gallon Finished Volume |
| Monoglycerides | 3500027 | 26 lbs |
| SAPP | 3500011 | 4.3 lbs |
| Citric Acid | 3500004 | 0.33 lbs |
| Tocopherols | 3500006 | 0.22 lbs |
| Tank #2 Sulfite |  | 100 Gallon Finished Volume |
| Sodium Metabisulfite | 3500026 | 25 lbs. |

| Production Feed Rates – LEWISVILLE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Cook/Cool Slabs | (wet lbs./min) | 290 | 300 | 310 | 315 | 320 |
| Tank #1 Emulsifier Flow Rate | (mL/min) | 3480 | 3600 | 3720 | 3780 | 3840 |
| Tank #1 Emulsifier Flow Rate Range | (mL/min) | +/- 200 mL/min |  |  |  |  |
| Tank #2 Sulfite Flow Rate | (mL/min) | 580 | 600 | 620 | 630 | 640 |
| Tank #2 Sulfite Flow Rate Range | (mL/min) | +/- 50 mL/min |  |  |  |  |
| Finished Dry Product | (lbs./min) | 63 | 66 | 68 | 69 | 70 |

| RUPERT PLANT |  |  |
| --- | --- | --- |
| Additives | Item Number | Batch Weights |
| Tank #1 Emulsifier |  | 100 Gallon Finished Volume |
| Monoglycerides | 3500027 | 66 lbs |
| SAPP | 3500011 | 10.5 lbs |
| Citric Acid | 3500004 | 0.8 lbs |
| Tocopherols | 3500006 | 0.5 lbs |
| Tank #2 Sulfite |  | 100 Gallon Finished Volume |
| Sodium Metabisulfite | 3500026 | 15 lbs |

| Production Feed Rates – RUPERT PLANT |  |  |  |
| --- | --- | --- | --- |
| Production Line |  | LINE 1 | LINE 2 |
| Cooked/Cooled Slabs | (wet lbs/min) | 140-160 | 210-240 |
| Tank #1 Emulsifier Flow Rate | (mL/min) | 1200 | 1800 |
| Tank #1 Emulsifier Flow Rate Range | (mL/min) | +/- 200 mL/min |  |
| Tank #2 Sulfite Flow Rate | (mL/min) | 700 | 1000 |
| Tank #2 Sulfite Flow Rate Range | (mL/min) | +/- 100 mL/min |  |
| Finished Dry Product | (lbs/min) | 31-35 | 46-53 |

Production Notes:

- All calculations based on 20% solids.

- For emulsifier preparation in low shear mixers. Fill additive tank approximately 2/3 full with water.  Heat to 150F and add all ingredients.  Mix until the monoglycerides are completely dissolved, approximately 15 minutes.  Fill remainder of tank with water to the fill line and allow approximately 10 minutes of mixing before draining to lower tank.

V.	FINISHED PRODUCT REQUIREMENTS

- Dry Product

| Physical & Chemical Attributes | LSL | Target | USL |
| --- | --- | --- | --- |
| Color | Shall possess a bright, uniform, potato color which may vary from light cream to pale yellow. |  |  |
| Defects per 100 grams |  |  |  |
| Minor (Less than 1/8” and greater than 1/16”) |  |  | 23 |
| Scorch |  |  | 5 |
| Majors (Greater than 1/8”) |  |  | 4 |
| Total Defects per 100 grams Total Defects = the sum of all Minor, Scorch, and Major defects. |  |  | 23 |
|  | Defects include but are not limited to grey, black, dark brown, scorch, green spots. A sample exceeding any single defect category is Out Of Spec. |  |  |
| Laminated Pieces/Shell (per 100 grams) |  |  | 3 |
| Tapped Bulk Index (mL/100 grams) | 330 | 360 | 380 |
| Screen Particle Size (100 g, 3 minutes) |  |  |  |
| - US#4 - ON PAN | 0% |  | 25% |
| - US#40 – ON PAN | 60% |  | 100% |
| - Base Pan | 0% |  | 15% |
| Moisture % (LSL not out of spec) | 7.0 | 8.5 | 9.0 |
| Sulfite (as SO2) ppm | 200 | 350 | 500 |
| Reducing Sugars % |  |  | 3.0 |
| Metal Ferrous (mm) Non-Ferrous (mm) Stainless (mm) |  |  | 2 2 2.5 |
| Foreign Material | None Detected |  |  |

| Microbiological Attributes | LSL | Target | USL |
| --- | --- | --- | --- |
| Standard Plate Count/g |  |  | 50,000 |
| Coliform/g |  |  | 10 |
| Yeast and Mold/g |  |  | 100 |

- Preparation (Texture)

- Weigh out 50 grams of flake.

- Add 250 ml of 160°F water.

- Stir to moisten.

- Let stand 1 minute.

- Use fork to stir gently (10-15 circular motions) for 10 seconds.

- Pick small amount of mashed potatoes and roll into a ball. Make a distinct break and      grade the texture 5-1 by the cleanliness of the break and how much the product bends following Procedure A.2 Recons.

- Preparation (Retail)

- Measure 2/3 cup (157 ml) water, ¼ tsp (1.25 grams) salt, and 1 tbsp (14 grams) butter into a 1 qt. pan. Bring to a boil.

- Remove from heat, add 1/3 cup (78 ml) cold 2% milk.

- Stir in 2/3 cup (44 grams) potato flakes until wet.

- Let stand 1 minute.

- Whip lightly with fork.

- Prepared Product

- Texture – Typical potato texture free from gumminess and pastiness. No loose standing water, no hard or dry lumps. Graded on 1-5 scale.

- Appearance – Bright, uniform, potato color which may vary from light creamy white to pale yellow. Graded on a 1-5 scale.

- Flavor and Aroma – Normal cooked potato flavor and aroma. Free from objectionable foreign flavor and aroma. Graded on a 1-5 scale.

| Prepared Product Attributes | LSL | Target |  | USL |
| --- | --- | --- | --- | --- |
| Texture | 3 | 5 | 5 |  |
| Appearance and Color | 3 | 5 | 5 |  |
| Flavor and Aroma | 3 | 5 | 5 |  |

VI.  	PACKAGING

Packaged in poly lined tote with a net weight of approximately 850 lbs.

VII.  	SHELF LIFE & STORAGE INFORMATION

The shelf life is approximately 12 months from date of production, when stored away from direct heat, in a cool, dry location, away from strong contaminating odors. Storage conditions should not exceed 80°F and 60% R.H.

VIII.	ALLERGENS

US - None      INTERNATIONAL – Sulfites

IX.  	CERTIFICATION

Kosher