PRODUCT SPECIFICATIONS FOR IDAHOAN®

CLEAN LABEL REAL WITH SALT MASHED POTATOES

WIP #2200005

Issued: October 7, 2025

Supersedes: May 2, 2023

I. 	PRODUCT DESCRIPTION

Idahoan® Clean Label REAL with salt Premium Mashed Potatoes are processed from sound, well peeled, and trimmed 100% Idaho® Potatoes. The product complies with the provisions of the Federal Food, Drug and Cosmetic Act and Amendments.

II.   	INGREDIENTS

Idaho® potatoes, salt, sunflower oil, nonfat dry milk, monoglycerides, calcium stearoyl lactylate, natural flavor. Freshness preserved by sodium acid pyrophosphate, sodium bisulfite, citric acid, and mixed tocopherols.

III.	PRODUCTION FEED RATES & TARGETS

| Components of 2200005 | Target | Range | QA Test Available |
| --- | --- | --- | --- |
| Mix Moisture | 32.5% | 30-35% | Yes |

| Production Feed Rates |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ingredient | Rate | Item | Added weight based on flake input weight |  |  |  |  |  |
| Flakes | lbs./min | 2100117 | 24 | 26 | 28 | 30 | 32 | 34 |
| Salt | grams/min | 3500001 | 415 | 450 | 484 | 519 | 554 | 588 |
| Nonfat Dry Milk | grams/min | 3500019 | 117 | 127 | 136 | 146 | 156 | 166 |
| Oil | grams/min | 3500021 | 234 | 253 | 273 | 292 | 312 | 331 |
| - Oil Volumetric | mL/min | 3500021 | 266 | 288 | 310 | 332 | 354 | 377 |
| Nat. Butter FL | grams/min | 3600003 | 11.7 | 12.7 | 13.6 | 14.6 | 15.6 | 16.6 |
| Clean Label Premix | grams/min | 2110012 | 33 | 35 | 38 | 41 | 44 | 46 |
| Total Produced Weight | lbs./min | 2200005 | 25.8 | 27.9 | 30.1 | 32.2 | 34.4 | 36.5 |

| Production Feed Rates, cont. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ingredient | Rate | Item | Added weight based on flake input weight |  |  |  |  |  |
| Flakes | lbs./min | 2100117 | 50 | 55 | 60 | 65 | 70 | 75 |
| Salt | grams/min | 3500001 | 865 | 952 | 1038 | 1125 | 1211 | 1298 |
| Nonfat Dry Milk | grams/min | 3500019 | 244 | 268 | 292 | 317 | 341 | 366 |
| Oil | grams/min | 3500021 | 487 | 536 | 585 | 634 | 682 | 731 |
| - Oil Volumetric | mL/min | 3500021 | 554 | 609 | 665 | 720 | 775 | 831 |
| Nat. Butter FL | grams/min | 3600003 | 24 | 27 | 29 | 32 | 34 | 37 |
| Clean Label Premix | grams/min | 2110012 | 68 | 75 | 82 | 89 | 96 | 102 |
| Total Produced Weight | lbs./min | 2200005 | 53.7 | 59.1 | 64.5 | 69.8 | 75.2 | 80.6 |

| Ingredient Operating Range |  |  |  |
| --- | --- | --- | --- |
| Ingredient | Rate | Item | Operating Range |
| Flakes | lbs./min | 2100117 | +/- 0.5 lbs. |
| Salt | grams/minute | 3500001 | +/- 20 grams to Meet Spec |
| Nonfat Dry Milk | grams/minute | 3500019 | +/- 10 grams |
| Oil | grams/minute | 3500021 | +/- 20 grams |
| - Oil Volumetric | mL/min | 3500021 | +/- 25 mL |
| Nat. Butter FL | grams/minute | 3600003 | +/- 1 grams |
| Clean Label Premix | grams/minute | 2110012 | +/- 2 grams |

Production Notes:

- All calculations based on 20% solids

- No colored water added

IV. FINISHED PRODUCT REQUIREMENTS

- Dry Product

| Physical & Chemical Attributes | LSL | Target | USL |
| --- | --- | --- | --- |
| Color | Shall possess a bright, uniform, potato color which may vary from light cream to a pale yellow. |  |  |
| Defects per 100 grams |  |  |  |
| Minor Defects (Greater than 1/16” to less than 1/8”) |  |  | 15 |
| Scorch |  |  | 5 |
| Major Defects (Greater than 1/8”) |  |  | 4 |
| Total Defects per 100 grams Total Defects = the sum of all Minor Defects, Scorch, Green, and Major Defects. |  |  | 15 |
|  | Not more than 15 defects. Defects will include but are not limited to grey, black, dark brown, scorch, or green spots. A sample exceeding any single defect category is Out Of Spec. |  |  |
| Bulk Index (ml/100 grams) | 230 | 255 | 280 |
| Moisture % (LSL not out of grade) | 7.0 | 8.0 | 9.0 |
| Sulfite (as SO2 ppm) | 200 | 350 | 500 |
| Salt % | 3.30 | 3.55 | 3.8 |
| Oil % without Monoglycerides (Record Only) | 1.6 | 2.0 | 2.6 |
| Metal Ferrous (mm) NonFerrous (mm) Stainless (mm) |  |  | 2 2 2.5 |
| Foreign Material | None Detected |  |  |

| Microbiological Attributes | LSL | Target | USL |
| --- | --- | --- | --- |
| Standard Plate Count/g |  |  | 50,000 |
| Coliform/g |  |  | 50 |
| Yeast and Mold/g |  |  | 100 |

B.  	Preparation

1.  	Weigh out 356 grams of product.

2.  	Measure ½ gallon water (minimum of 195°F) into a steam table pan and add product while stirring with wire whip until all potatoes are wet.

3.  	Let stand one minute and then fluff.

C.  	Prepared Product

1.  	Color – typical, cooked, rich, potato color ranging from light cream to creamy yellow.  No objectionable darkening 2 hours after preparation. Graded on 1-5 scale.

2.  	Texture – typical, creamy, fluffy, mashed potato texture-free from gumminess and pastiness.  Graded on 1-5 scale. No puddling and no more than three lumps per #8 scoop.

3.  	Flavor and Aroma – normal, cooked, potato flavor and aroma with light buttery flavor and aroma.  Free from objectionable foreign flavors and odors. Graded on 1-5 scale.

4.  	Steamtable – 4 hours no significant change in flavor, texture, color or aroma. Graded on 1-5 scale.

| Prepared Product Attributes | LSL | Target |  | USL |
| --- | --- | --- | --- | --- |
| Color | 3 | 5 | 5 |  |
| Texture | 3 | 5 | 5 |  |
| Lumps (per #8 scoop) | 0 | 0 | 3 |  |
| Flavor and Aroma | 3 | 5 | 5 |  |
| Steamtable (4 hours) | 3 | 5 | 5 |  |

V.  	PACKAGING

Product is packaged in a poly-lined tote.

VI.  	SHELF LIFE & STORAGE INFORMATION

The shelf life is approximately 12 months from date of production when stored away from direct heat, in a cool, dry location, away from strong contaminating odors. Storage conditions should not exceed 80°F and 60% R.H.

VII.	ALLERGENS

US – Milk        INTERNATIONAL – Milk, Sulfites

VIII.   CERTIFICATION

Kosher Dairy